Submitted By:

Uqba Gulzar

Submitted To:

Engr. Muhammad Shehzad

Reg No:

2023-BSE-067

Section:

BSE-IVB

Department:

Software Engineering

Course Title:

CCN

LAB 05:

Understand the basic commands for switch port security on different VLAN environment

according to Figure 1.

Switch1>enable

Basic Switch Commands:

Switch1#Show vlan brief (showing interfaces of vlan briefly)

Making VLANS

Switch1#config terminal

VlAN 10  name Actg   exit  vlan 20   name Mkt   exit  vlan 30  name Hr

exit

switch1#show vlan brief

Assigning Ethernet Ports to VLANs:

Switch#config t

Switch(config)#interface range fa0/1-3

Switch(config-if)#

Switch(config-if)#switchport mode access

Switch(config-if)#switchport access vlan 10

Switch(config)#interface range fa0/4-5

Switch(config-if)#switchport mode access

Switch(config-if)#switchport access vlan 20

Switch(config)#interface range fa0/6-7

Switch(config-if-range)#switchport mode access

Switch(config-if-range)#switchport access vlan 50